# ROBOMANIPAL TASKPHASE  2

sensor is a device or module that detects changes in physical or environmental, chemical, or biological quantities—such as temperature, pressure, light, motion, or electrical properties—and converts these changes into signals that can be measured or interpreted by electronic systems. This sensor is based on the functions of the human sensory organs. Robots receive a broad range of data about their surroundings, such as position, size, orientation, velocity, distance, temperature, weight, force, etc. This information is what allows the robot to function efficiently while interacting with its environment to perform complex tasks.

# TYPES OF SENSORS-

## PROXIMITY SENSORS-

These are type of sensors, a device that can detect or sense the approach or presence of nearby objects and for this it does not need physical contact. A proximity sensor often emits an electromagnetic field or a beam of electromagnetic radiation (infrared, for instance) and looks for changes in the field or return signal. The object being sensed is often referred to as the proximity sensor's target.

### Types of proximity sensors-

- Inductive Proximity Sensors: Detect metallic objects using high frequency alternating magnetic field. A coil generates an AC field; when a metal object enters, eddy currents change the coil’s impedance, triggering a signal. Commonly used in manufacturing for detecting metal parts. Can sense up to 30 mm.

Components: oscillator/coil, detection electronics, comparator/Schmitt trigger, housing (IP-rated), output transistor (NPN/PNP).

Adv-robust, immune to dust/lighting

Dis adv-only detects metal.

Applications: metal part detection on conveyors, limit switches, position sensing in machines.

- Capacitive Proximity Sensors: Detect both metallic and non-metallic objects. These work like two parallel capacitor plates—one is the sensor; the other is the target. When a target approaches, it changes the capacitance, which the sensor detects. Applications include fluid level detection and packaging lines. Senses up to 50 mm.

- Magnetic Proximity Sensors: Use magnets and reed switches, detecting the presence of a magnetic field. Used in door sensors and robotics for position detection.

- Ultrasonic Proximity Sensors: Use high-frequency sound waves. The sensor emits a sound pulse, and when it bounces off an object, the echo time helps determine presence and distance. Used for obstacle detection, automation, and robotics. range is about few cm .

- Optical/Photoelectric Proximity Sensors: Include a light emitter (LED or laser) and a receiver. When an object interrupts or reflects the light path, the sensor triggers. Applications: conveyor belts, safety systems.

Components: Depending on the type—coils, capacitors, magnets, ultrasonic transducers, LEDs, photodetectors, signal amplifiers, and control circuits

## COLOUR SENSOR-

These sensors detect the colour (typically by measuring red, green, blue light intensities) and sometimes ambient light or proximity.

- Working Principle: Typically uses an array of photodiodes or phototransistors with colour filters (red, green, blue). When light reflects off the target and passes through these filters, the sensor generates signals proportional to the intensities of the RGB components. Photodiode array + integrated ADC + I²C/SPI chip (e.g., TCS3472-like architectures) provides digital RGB counts and can return colour temperature and lux.

- Components-Photodiodes or phototransistors, colour filters (R/G/B), optional IR blocking filter, controlled white LED for consistent illumination, ADC, microcontroller/IC with interface (I²C/SPI), lens/window.

- Application-Colour sorting in manufacturing, colour detection in consumer devices (printers), ambient colour sensing, product inspection, line-tracking by colour.

## LDR (Light Dependent Resistor / Photoresistor)-

An LDR, or photoresistor, is a variable resistor whose resistance decreases as light intensity increases.

- Working Principle: Made from typically cadmium sulphide (CdS) or similar semiconducting compounds. When exposed to light, photons increase the number of free electrons, decreasing resistance. serpentine film of photo resistive material deposited on ceramic substrate, leads attached, sealed under glass or epoxy.

- Ability- Dark resistance: megaohms, illuminated resistance: hundreds of ohms depends on light intensity and device.  Response time slower than photodiodes/phototransistors Temperature-dependent and long-term drift possible.

- Components: Cadmium sulphide (CdS) or similar semiconductor substrate, electrodes.

- Applications: Automatic street lighting, light/dark sensors for alarms, camera exposure controls, and solar garden lights.

## HALL EFFECT SENSORS-

detects magnetic field strength (proportional measurement) or presence/polarity of magnets (switching). A Hall-effect sensor, also known as a Hall sensor, monitors magnetic fields with high accuracy, consistency and reliability. It is that important Because it enables you to sense the position and movement of objects in a system. A voltage is generated transverse to the current flow in a semiconductor or conductor when a magnetic field is applied perpendicular to the current.

- Working Principle: Movement of charge carriers in a magnetic field creates a Hall voltage correlated to field strength. when a current flows through a conductor/semiconductor in a magnetic field perpendicular to the current, a transverse voltage (Hall voltage) appears across the device proportional to magnetic flux density.

- Components: Hall-effect element (thin semiconductor plate), bias circuitry, amplifier, comparator, temperature compensation, output stage (open-drain or push-pull), packaging with mounting holes.

- Applications: Position detection (rotary or linear), speed sensing (e.g., tachometers), current sensing in motor controllers, automotive ignition systems, and brushless motor commutation.

## IMU (Inertial Measurement Unit)

An Inertial Measurement Unit (IMU) is an electronic device that measures and reports a body’s specific force, angular rate, and sometimes the magnetic field surrounding the body, using a combination of accelerometers, gyroscopes, and sometimes magnetometers.

- Working Principle: Combines accelerometers (measuring linear acceleration) and gyroscopes (measuring rotational velocity); advanced IMUs include magnetometers for orientation referencing.

- Components: MEMS accelerometers, MEMS gyroscopes, sometimes magnetometers, signal conditioning circuits, and digital interfaces.

- Applications: Drones, robotics navigation, automotive stability controls, mobile phones (orientation detection), aerospace systems for stabilizing and guidance.

## ENCODERS-

Encoders convert motion (linear or rotary) into electrical signals for position, speed, or direction determination.

### Working Principle:

  - Optical Encoders: A slotted or patterned disc interrupts a light beam between a LED and photodetector; the resulting pulses correspond to movement or rotation.

  - Magnetic Encoders: Use magnetic fields and magneto resistive sensors to achieve similar effects, especially in harsh environments.

- Components: Rotating code disc (optical type), LED, photodetectors, signal processing circuitry, fixed scale (for linear encoders).

- Applications: Industrial automation for motion feedback, robotics for joint position, CNC machines, printers, and motor controllers.

## TEMPERATURE SENSOR-

A temperature sensor is a device that measures temperature by converting thermal energy into an electrical signal. It works by changing a physical property, such as resistance or voltage, in response to temperature fluctuations, allowing it to be used in applications from thermometers to refrigerators, air conditioners, and vehicle engines.

- Working: Measures temperature using either changes in resistance (thermistors, RTDs) or voltage (thermocouples) as temperature varies.

- Components: Sensing element (thermistor, thermocouple, RTD), sometimes a reference junction, signal conditioning electronics.

- Applications: HVAC systems, automotive engines, household appliances, digital thermometers, industrial process monitoring

## PRESSURE SENSOR-

A pressure sensor is a device that measures pressure in liquids or gases and converts it into a usable electrical signal for monitoring and control. It works by detecting the deformation of a diaphragm or membrane under applied pressure, then using technologies like piezoresistive, capacitive, or piezoelectric principles to translate this physical change into an electrical output proportional to the pressure.

- Working: Converts applied pressure (from liquid or gas) to an electrical signal, often using piezoresistive, capacitive, or optical methods.  Pressure deforms a diaphragm → changes resistance/capacitance/voltage (depending on sensor type).

- Components: Diaphragm, sensor elements (piezoresistive, capacitive plates), signal conditioning circuit.

- Applications: Tire pressure monitoring, barometers, industrial process control, blood pressure monitors, weather stations

## ACCELEROMETER-

An accelerometer sensor measures changes in velocity, or acceleration, including static forces like gravity and dynamic forces from movement or vibrations. It converts mechanical motion into an electrical signal, allowing devices to sense orientation and movement.

- Working: Measures acceleration using a suspended mass whose displacement (due to acceleration) is sensed via changes in capacitance or resistance.

- Components: Suspended mass (proof mass), spring, capacitance or piezoresistive sensors, signal processing circuit.

- Applications: Airbag deployment in vehicles, smartphone orientation, vibration detection, motion tracking in robotics and gaming.

## ULTRASONIC SENSOR-

An ultrasonic sensor is a device that uses high-frequency sound waves (ultrasound) to measure the distance to an object by detecting the "echo" of the sound wave after it bounces off the object.

- working: Emits ultrasonic sound waves and measures the time for the echo to return after reflection, calculating distance based on time-of-flight.

Transmits ultrasonic pulse → reflects from object → echo received → distance calculated from time-of-flight

- Components: Ultrasonic transducer (for emitting/receiving sound), timing circuitry, microcontroller for signal processing.

- Applications: Distance measurement, obstacle detection for robots and vehicles, liquid level measurement, parking assistance systems.

## HUMIDITY SENSOR-

A humidity sensor, or hygrometer, is an electronic device that measures the amount of water vapor in the air, or humidity. It functions by changing its electrical properties—such as capacitance or resistance—in response to moisture, converting this change into an electrical signal that can be read and processed.

- Working: Detects changes in humidity by measuring changes in electrical capacitance or resistance of a hygroscopic material as it absorbs moisture.

- Components: Hygroscopic substrate (polymer, ceramic), electrodes, sometimes integrated temperature sensor, signal processing circuit.

- Applications: Environmental monitoring, weather stations, HVAC systems, industrial process control, agricultural monitoring.

## IR (INFRARED) SENSOR-

A sensor that detects infrared radiation from objects or uses IR light reflection to sense proximity, heat, or motion.

- Working: Active type → IR LED emits light, photodiode detects reflection. Passive type → detects ambient IR (heat).

- Components: IR LED (emitter), photodiode/phototransistor (receiver), filters, processing circuit.

- Applications: Remote controls, flame detectors, night vision systems, proximity sensors in phones.